Nikon D7100 Camera Settings

Shutter Priority:  “S” on top main round dial

Multiple Pictures:  CL on ring under main round dial

Focus: small rectangle should be centered in viewfinder

- to fix: ring around rear ok button to white dot

- move dot with navigation ring around ok button

- ring around ok button to “L”

Menu Button (rear, upper left)

- Shooting Menu (camera icon)

- ISO Sensitivity Settings: 125/ON/2500/Auto

- Setup Menu (wrench icon)

- Format memory card

- Time Zone and Date

- GPS  - use to set camera clock

- Wireless Mobile Adapter - off

Set ISO-Auto mode

- hold ISO button (rear left, 2nd from bottom)
    and rotate wheel on front under power,

watch display on top
Shutter Speed: rear thumb wheel, 2000 min

Picture Review
- Press rear top left button (right-pointing triangle)
- Zoom: press the magnifying glass icons on rear left